Nurbek Abdulgazimov

230103215

Host CPU / SoC Model: 12th Gen Intel Core i7-12650H

Operating System: Windows 10 Home, 64-bit

Task 1:

CPU Model & Architecture - 12th Gen Intel Core i7-12650H

Base & Max Boost Clock - 2.30 GHz / 4.70 GHz

Physical Hardware Cores - 10

Logical Cores (SMT/Threads) - 16

L1 Data / Instruction Cache - L1d: 416 KB / L1i: 448 KB

L2 Cache - 9.5 MB total

L3 Shared Last-Level Cache - 24 MB

SIMD Vector Extensions - AVX, AVX2

Q1.1: Flynn’s Taxonomy Mapping

My CPU can work as SISD when one thread runs instructions sequentially. It can use SIMD with AVX and AVX2 to process multiple data values at once. It can also work as MIMD because its multiple CPU cores can run different tasks at the same time.

Task 2:

Workload Algorithm: Prime counting up to 5,000,000

Language & Runtime: Python 3.12.10 / multiprocessing (ProcessPoolExecutor)

| N | Run 1 (s) | Run 2 (s) | Run 3 (s) | Avg Time TN (s) | Speedup SN | Efficiency EN |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 12.136 | 12.258 | 12.744 | 12.379 | 1.00× | 100.0% |
| 2 | 11.232 | 8.351 | 8.108 | 9.230 | 1.34× | 67.1% |
| 4 | 4.960 | 4.931 | 5.213 | 5.035 | 2.46× | 61.5% |
| 8 | 3.582 | 3.618 | 3.802 | 3.667 | 3.38× | 42.2% |
| 16 | 2.659 | 2.457 | 2.363 | 2.493 | 4.97× | 31.0% |
| 32 | 2.264 | 2.379 | 2.548 | 2.397 | 5.16× | 16.1% |

Q2.1: Scaling Saturation & Hardware Contention Analysis

The benchmark scales well at first, but the performance starts to saturate around 16 workers. My CPU has 10 physical cores and 16 logical processors. Increasing the number of workers from 16 to 32 only improves the average time from 2.493 s to 2.397 s. This happens because 32 workers are more than the available logical processors, so the operating system has to schedule and switch between them. CPU resource sharing and cache contention also reduce the parallel efficiency.