Parallel Computing Laboratory Report

| Field | Value |
| --- | --- |
| Student Full Name | Nurbek Abdugazimov |
| Student ID | 230103215 |
| Date | 18 September 2026 |
| Laptop | MacBook Air |
| Processor | Apple M1 |
| CPU Topology | 8 physical cores: 4 Performance + 4 Efficiency; 8 logical cores |
| Cache | L1 I: 128 KB; L1 D: 64 KB; L2: 4 MB; L3: not reported |
| Memory | 8 GB LPDDR4 Unified Memory |
| OS / Python | macOS 15.6.1 (24G90) / Python 3.12.2 |

# Task 1. Amdahl Scaling

| Workers | Time (s) | Speedup | Ideal | Efficiency |
| --- | --- | --- | --- | --- |
| 1 | 2.2027 | 1.00x | 1.00x | 100.0% |
| 2 | 1.1947 | 1.84x | 2.00x | 92.2% |
| 4 | 0.7018 | 3.14x | 4.00x | 78.5% |
| 8 | 0.5172 | 4.26x | 8.00x | 53.2% |
| 12 | 0.5368 | 4.10x | 12.00x | 34.2% |
| 16 | 0.5740 | 3.84x | 16.00x | 24.0% |

Q1.1. Maximum speedup occurs at 8 workers (4.26x). After 8 workers performance decreases because the Apple M1 has 8 physical cores and additional processes increase scheduling overhead.

Q1.2. S(4) = 2.2027 / 0.7018 = 3.1386. From Amdahl's Law, P ≈ 0.9085, so the parallel fraction is about 90.85%.

Q1.3. Smax = 1 / (1 - 0.9085) ≈ 10.93x. A 128-core system would still have strong diminishing returns for this workload.

# Task 2. Cache Coherence and False Sharing

| Configuration | Trial 1 | Trial 2 | Trial 3 | Median |
| --- | --- | --- | --- | --- |
| Adjacent indices | 0.7271 | 0.7277 | 0.7283 | 0.7277 |
| Padded indices | 0.7271 | 0.7269 | 0.7309 | 0.7271 |

Q2.1. A 64-bit CPython list reference is 8 bytes. A stride of 16 separates slots by 128 bytes, which is larger than a 64-byte cache line. The measured slowdown was approximately 1.00x because the GIL largely serialized the Python threads.

Q2.2. When two cores write different values on the same cache line, ownership moves between the cores through MESI invalidations. Repeated transfers create cache-line ping-pong and false-sharing stalls.

Q2.3. Common solutions include C/C++ alignas(64) and Java @Contended, which separate frequently written variables onto different cache lines.

# Task 3. Synchronization and Lockless Redesign

| Architecture | Actual | Time (s) | Result |
| --- | --- | --- | --- |
| UnsafeCounter | 2,000,000 | 0.1064 | Baseline |
| LockedCounter | 2,000,000 | 0.4089 | 3.84x slower |
| Lockless thread-local | 2,000,000 | 0.0529 | 7.73x faster than locked |

Q3.1. UnsafeCounter returned the correct value in this run, so the observed defect count was 0. A race can still occur when multiple threads overlap the LOAD → ADD → STORE sequence, but the CPython GIL prevented it from appearing here.

Q3.2. The lockless version used one private accumulator per thread and a final reduction. It returned exactly 2,000,000 in 0.0529 s, giving a 7.73x speedup over LockedCounter.

import threading, time

TOTAL_OPS = 2_000_000

NUM_THREADS = 4

def lockless_bench():

ops = TOTAL_OPS // NUM_THREADS

partial = [0] * NUM_THREADS

def work(i):

local_sum = 0

for _ in range(ops):

local_sum += 1

partial[i] = local_sum

threads = [threading.Thread(target=work, args=(i,)) for i in range(NUM_THREADS)]

start = time.perf_counter()

for t in threads: t.start()

for t in threads: t.join()

return sum(partial), time.perf_counter() - start

Q3.3. Thread-local state removes repeated mutex operations and shared-state contention. Synchronization is needed only for the final reduction, so the approach is faster and more scalable.

# Task 4. Memory Wall vs. Compute Saturation

| Workers | Compute (s) | Compute Tw/T1 | Memory (s) | Memory Tw/T1 |
| --- | --- | --- | --- | --- |
| 1 | 0.8606 | 1.00x | 0.5227 | 1.00x |
| 2 | 1.0791 | 1.25x | 0.6957 | 1.33x |
| 4 | 1.2008 | 1.40x | 1.4791 | 2.83x |

Q4.1. At 4 workers, compute time increased only 1.40x, while memory time increased 2.83x. Memory-heavy workers compete for shared memory bandwidth, so scaling becomes worse.

Q4.2. The M1 system has 8 GB unified memory and a theoretical peak memory bandwidth of about 68.25 GB/s. Shared memory bandwidth limits multicore scalability for data-intensive workloads.

Q4.3. For a memory-bandwidth-limited workload, adding more CPU threads gives limited benefit. A high-bandwidth GPU can help when data remains on the GPU. Apple M1 uses Metal rather than CUDA.